- Pipe, Schedule 80, 6 In, 10 ft. Length, PVC

- HARVEL

- Zoro #: G5276056  Mfr #: H0800600PG1000

- $167.20

In stock

This item ships FREE.

- Pipe, PVC, 4 In, Schedule 80, 10 Feet L

- GF PIPING SYSTEMS

- Zoro #: G3064871  Mfr #: H0800400PG1000

- $82.49

  - Pipe, PVC, 3 In, Schedule 80, 10 Feet L

  - GF PIPING SYSTEMS

  - Zoro #: G1528012  Mfr #: H0800300PG1000

  - $66.72

In stock

This item ships FREE.

Top of Form

Add to Cart

Bottom of Form

In stock

This item ships FREE.

- Pipe, PVC, 2 1/2 In, Schedule 80, 10 Feet L

- GF PIPING SYSTEMS

- Zoro #: G1367116  Mfr #: H0800250PG1000

- $64.33

In stock

This item ships FREE.

- Pipe, 2 In, PVC, Schedule 80, 10 Ft L

- GF PIPING SYSTEMS

- Zoro #: G2615812  Mfr #: H0800200PG1000

- $23.99

In stock

PVC 80

6” -80	6.625	5.708	0.432	Zoro #: G5276056  Mfr #: H0800600PG1000 	$167.20	56 lbs

5” -80	5.563	4.768	0.375

4” -80	4.5	3.786	0.337	Zoro #: G3064871  Mfr #: H0800400PG1000	$  82.49 	29 lbs

3” -80	3.5	2.29	0.276	Zoro #: G1528012  Mfr #: H0800300PG1000 	$  66.72	20

2” -80	2.375	1.913	0.218	Zoro #: G2615812  Mfr #: H0800200PG1000 	$  23.99	10

Orders over $50 ship FREE.

(Make 3 orders to avoid lift gate service )

add aluminum flashing?

Schedule 80 PVC pipe diameters (nested 6”, 5”, 4”, 3”, 2”)

Scale: actual size

dielectric  strength of PVC pipe is about 14-20kV/mm

500 kV /`(15kV/mm) = 33.3 millimeters

use aluminum flashing and electrode grading

use refrigeration oil (for dryness)

add Drierite cylindrical shell to core.

various wooden and plastic plugs

calculate capacitance and impedance

Calculated capacitance

Calculated capacitance of one 4 ft active section of 4” dia. Schedule 80 PVC pipe (5 ft total length).

 r =  4.5

A:

(3.14)(3.78)(48) = 570 in2

570 in2  (1 m2/1550 in2)

= 0.368 m2

d:

0.377 in  (1m/ 39.4 in)

=9.57 x 10-3 m

Hence:

(8.854×10−12 F m–1)(4.5)(0.368/9.57 x 10-3)

=1.53 x 10-12 x 103 Farads

=1.53 x 10-9 Farads

Calculated energy storage at 200,000 volts:

W = 1/2CV2

= (0.5)( 1.53 x 10-9 )(2 x 105)2

= 3.06 x 101

= 30 joules

This is enough energy to light a 30 watt light bulb for 1 second. However, the energy is released as a pulse. If the pulse is 0.1 microseconds long, the instantaneous power level is 300 million watts.

Side view PVC capacitor main tubes

Scale:  1:7

Schedule 80 PVC specs.:

4” 	4.5”     OD	3.786”   ID      .643     0.541

3” -	3.5”     OD	2.864”   ID      .500     0.409

2” -	2.375” OD	1.913”   ID     .340     0.273

60”

12”

10”

14”

15”

.34

stainless steel wool contact

15”

14”

10”

12”

PVC pipe lengths are 60inches; cut from 10 ft lengths; when ordering, group so as to avoid lift gate service charge

Electrode lengths are 40, 36, 32, and 30 inches long. Width to be determined at assembly.

Annulus between 2” and 3” pipe has two electrode surfaces and an annular fill with barium titanate (on-hand) to reduce dielectric stress

Ends are closed with PVC slip caps and dielectric grease. Bottom end has a nylon plug.

Top end has a copper wire sheathed with 3 layers of concentric clear PVC tubing .

End view

Scale: actual size

Aluminum electrode (red)

PCV schedule 80 (blue)

4”

air annulus (white)

3”

2”

Barium titanate filled annulus (orange)

high voltage electrode

Calculated impedance

http://www.allaboutcircuits.com/tools/coax-impedance-calculator/

Where:

Z0  	Z0  = characteristic impedance in ohms (Ω)

td  	td  = time delay in nanoseconds per inch (ns/in)

L	L  = inductance per unit length in nanohenries per inch (nH/in)

C	C  = capacitance per unit length in picofarad per inch (pF/in)

d2	d2  = inner surface shield diameter

d1	d1  = inner conductor diameter